### Raspberry Pi getting data from a S7-1200 PLC

UPDATE: If you want the raspberry pi to be the s7 server go here
UPDATE 2: If you want to see communication with S7-200 go here
UPDATE 3: Video walkthrough on setup go here

I recently borrowed a S7-1200 PLC from work to see if I could get data from it using a Raspberry Pi. In my search for something I found that Snap7 was the best option.

Steps to getting it work

- Download and compile snap7 (http://sourceforge.net/projects/snap7/files/1.2.1/snap7-full-1.2.1.tar.gz/download)

- Download and install python library to use snap7 (https://pypi.python.org/pypi/python-snap7)

#download and compile snap7 for rpi

wget http://sourceforge.net/projects/snap7/files/1.2.1/snap7-full-1.2.1.tar.gz/download

tar -zxvf snap7-full-1.2.1.tar.gz

cd snap7-full-1.2.1/build/unix

sudo make –f arm_v6_linux.mk all

#copy compiled library to your lib directories

sudo cp ../bin/arm_v6-linux/libsnap7.so /usr/lib/libsnap7.so

sudo cp ../bin/arm_v6-linux/libsnap7.so /usr/local/lib/libsnap7.so

#install python pip if you don't have it:

sudo apt-get install python-pip

sudo pip install python-snap7

You will need to edit the lib_location on common.py in the /usr/local/lib/python2.7/dist-packages/snap7/ directory
Add a line in the __init__ part of the Snap7Library class:
lib_location='/usr/local/lib/libsnap7.so' 
example below:

class Snap7Library(object):

"""

Snap7 loader and encapsulator. We make this a singleton to make

sure the library is loaded only once.

"""

_instance = None

def __new__(cls, *args, **kwargs):

if not cls._instance:

cls._instance = object.__new__(cls)

cls._instance.lib_location = None

cls._instance.cdll = None

return cls._instance

def __init__(self, lib_location=None):

lib_location='/usr/local/lib/libsnap7.so' # add this line here

if self.cdll:

return

self.lib_location = lib_location or self.lib_location or find_library('snap7')

if not self.lib_location:

msg = "can't find snap7 library. If installed, try running ldconfig"

raise Snap7Exception(msg)

self.cdll = cdll.LoadLibrary(self.lib_location)

Now you can write your client code :-)
Here's an example on how to connect and read an output Q0.0:

from time import sleep

import snap7

from snap7.util import *

import struct

plc = snap7.client.Client()

plc.connect("192.168.12.73",0,1)

area = 0x82    # area for Q memory

start = 0      # location we are going to start the read

length = 1     # length in bytes of the read

bit = 0        # which bit in the Q memory byte we are reading

byte = plc.read_area(area,0,start,length)

print "Q0.0:",get_bool(mbyte,0,bit)

plc.disconnect()

I created a helper class on my github here to make the syntax easier for people who are used to DAServer and Ladder:
https://github.com/SimplyAutomationized/raspberrypi/raw/master/S7-1200pi/S71200.py
Example on how to use it:

import S71200

from time import sleep

import snap7

from snap7.util import *

import struct

plc = S71200.S71200("192.168.21.65")

plc.writeMem('QX0.0',True) # write Q0.0 to be true, which will only turn on the output if it isn't connected to any rung in your ladder code

print plc.getMem('MX0.1') # read memory bit M0.1

print plc.getMem('IX0.0') # read input bit I0.0

print plc.getMem("FREAL100") # read real from MD100

print plc.getMem("MW20") # read int word from MW20

print plc.getMem("MB24",254) # write to MB24 the value 254

plc.plc.disconnect()

Let me know if there are questions. Hope I can help :-)
Also let me know if you can help me clean up my S71200.py helper class. I know it looks messy.

Follow me to get updates on a Raspberry pi Sensor the DA or OPC server can get data using S7 protocol.
+Simply Automationized

##### Check out my other SCADA posts

- Modbus TCP and Raspberry Pi Temperature Sensor

- Controlling Outputs Using Modbus TCP

at 9:40 PM

Email ThisBlogThis!Share to TwitterShare to FacebookShare to Pinterest

Labels: Pi, PLC, SCADA

#### 26 comments:

UnknownFebruary 21, 2019 at 1:29 AM

sudo make –f arm_v6_linux.mk all when I am running this line, I got the error like this

pi@raspberrypi:~/Downloads/snap7-full-1.2.1/build/unix $ sudo make –f arm_v6_linux.mk all
make: *** No rule to make target '–f'. Stop.

Please help me to overcome that error.thanks in advance.

Reply

Replies

A LifeJuly 3, 2019 at 12:55 PM

Hi, Please use the following instead:

sudo make -f arm_v6_linux.mk

aicomingsSeptember 23, 2021 at 5:30 AM

the "-f" should be written by yourself.

Reply

UnknownFebruary 27, 2019 at 7:37 AM

Don't copy the line, write yourself the line. (Sorry for my english, is very basic.)

Reply

Mani RamFebruary 27, 2019 at 12:55 PM

This comment has been removed by the author.

Reply

Mani RamFebruary 27, 2019 at 12:56 PM

I too have the same error for the line "sudo make –f arm_v6_linux.mk all"
Help me to overcome this problem.

Reply

Replies

NateFebruary 27, 2019 at 1:07 PM

you probably are using a Raspberry Pi 3.. which would be
"sudo make -f arm_v7_linux.mk all"

Reply

Mani RamFebruary 27, 2019 at 1:08 PM

i got it work, thanks :)

Reply

SadeMarch 7, 2019 at 9:51 AM

This comment has been removed by the author.

Reply

UnknownMarch 19, 2019 at 3:20 AM

Hello. I try fix allow above. But dont success. Can you help me!
snap7.snap7exceptions.Snap7Exception: can't find snap7 library. If installed, try running ldconfig

Reply

Nilanj PatelMarch 25, 2019 at 2:09 AM

i am not able to read address starting from "MW" of S71200 plc, but can read address starting from "MD" why is that so please help me.

Reply

houssem bahriApril 5, 2019 at 7:48 AM

How can i solve this problem
In [19]: plc.connect("192.168.0.1",0,1)
---------------------------------------------------------------------------
Snap7Exception Traceback (most recent call last)
in ()
----> 1 plc.connect("192.168.0.1",0,1)

/home/pi/.local/lib/python2.7/site-packages/snap7/client.pyc in f(*args, **kw)
23 def f(*args, **kw):
24 code = func(*args, **kw)
---> 25 check_error(code, context="client")
26 return f
27

/home/pi/.local/lib/python2.7/site-packages/snap7/common.pyc in check_error(code, context)
63 error = error_text(code, context)
64 logger.error(error)
---> 65 raise Snap7Exception(error)
66
67

Snap7Exception: TCP : Connection timed out

Reply

Replies

kdres1989June 12, 2020 at 10:59 PM

si lo lograste solucionar y como gRACIAS

Reply

UnknownMay 3, 2019 at 12:43 AM

when i try to install the snap7 lib i get this error i this line $ tar -zxvf snap7-full-1.2.1.tar.gz
tar (child): snap7-full-1.2.1.tar.gz : open impossible: Aucun fichier ou dossier de ce type
tar (child): Error is not recoverable: exiting now
tar: Child returned status 2
tar: Error is not recoverable: exiting now
what does it mean ?

Reply

UnknownMay 3, 2019 at 2:14 AM

hey , i installed the lib and when i tried to excute the code i got an error : no module named snap7 .
how can i fix this ?

Reply

jml69May 14, 2019 at 11:35 PM

in case of [tar -zxvf snap7-full-1.2.1.tar.gz] it was not working for me. Then found out the downloaded filename was [download.tar.gz], i needed to use [tar -zxvf download.tar.gz], by using [cd] / [dir] command first check the name of downloaded file, then execute the [tar -zxvf] command.

Reply

Souheil GhribiJuly 6, 2019 at 4:20 AM

How can i fix tis
File "plc2.py", line 1, in
import S71200
File "/home/pi/s7/S71200.py", line 110, in
plc = S71200('192.168.1.10') #,debug=True)
File "/home/pi/s7/S71200.py", line 16, in __init__
self.plc.connect(ip,0,1)
File "/usr/local/lib/python2.7/dist-packages/snap7/client.py", line 25, in f
check_error(code, context="client")
File "/usr/local/lib/python2.7/dist-packages/snap7/common.py", line 66, in check_error
raise Snap7Exception(error)
snap7.snap7exceptions.Snap7Exception: TCP : Unreachable peer

Reply

UnknownAugust 14, 2019 at 3:07 AM

i have 2 questions :
first one : how can we find the IP address that you wrote in your code [plc.connect("192.168...")] ?
and what is that IP address exactly ?
second one : do we connent raspberry to plc via Ethernet ?
Thanks

Reply

Replies

UnknownMay 30, 2021 at 1:35 AM

The ip address is the ip address of plc... You can set it up with tia portal (siemens automation software).... 2. Yes the snap7 uses ethernet to comunicate with the plc. It connects via "pg interface" (siemens administration protocol)... Of course you can setup an access point and access the plc via WiFi...

Reply

UnknownSeptember 11, 2019 at 1:49 AM

superr!! thank you for this .

Reply

UnknownSeptember 13, 2019 at 12:24 AM

It'able to get a howto with c++

Reply

UnknownOctober 7, 2020 at 12:42 AM

h

Reply

UnknownOctober 7, 2020 at 12:43 AM

how to get response ...on db_write or any write funtion.... that we know value write successfully in DBaddress????

Reply

ReachraoNovember 30, 2020 at 5:00 AM

Dear All,
i am trying to communicate S71200 PLC with RaspberryPI using Python program.

Following are the Pthon program and Error:
import S71200
from time import sleep
import snap7
from snap7.util import *
import struct

plc = S71200.S71200("192.168.43.2")
#plc.writeMem('QX0.0',True) # write Q0.0 to be true, which will only turn on the output if it isn't connected to any rung in your ladder code
print plc.writeMem('%DB1.QX4.0',False)# writing value
print plc.getMem("%DB1.DBX4.0") # read
print plc.getMem("%DB1.DBX8.0") # read
print plc.getMem("%DB1.DBW20") # read
plc.plc.disconnect()

Error:
>>> %Run Snap7_Example3.py
Traceback (most recent call last):
File "/home/pi/Desktop/Snap7_Example3.py", line 9
print plc.writeMem('QX4.0',False)# writing value
^
SyntaxError: invalid syntax
>>>



In SIEMENS TIA:
Address is: %DB1.DBX4.0
%DB1.DBX8.0
%DB1.DBD.16


Please advice

Reply

Replies

NakulFebruary 4, 2021 at 2:49 AM

Hi ReachRao,

It seems you are using Python3. The error here is showing invalid syntax as from Python3 print "abcd" is not supported and data needs to be kept in brackets.
For e.g. please change to print(plc.writeMem('QX4.0',False)) to resolve this error.

Reply

UnknownJune 22, 2021 at 12:31 AM

Thanks for a very detailed explaination, can you tell me were can i get the area for other plcs? or is it the same for all the S7 seires?

Reply

### Raspberry Pi SCADA Part 3: Communicate with the Pi using S7 Protocol.

I've been planning on trying this and have been able to get some success. In this example we are going to serve up temperature from the Raspberry Pi using Siemens S7 Protocol. I'm using Wonderware and their DAServer in my tests and will show images of the setup. 

For this example in Modbus TCP see here
To use the Pi to communicate with a PLC see here


First From a fresh Raspbian Image:

For Raspberry Pi 2 B add the following to the bottom boot/config.txt

#device tree config
dtoverlay=w1-gpio,gpiopin=4

#### Add the following to the /etc/modules

w1-gpio

w1-therm

#### Install python pip:

sudo apt-get install python-pip

#Download and build the latest snap7 library:

wget http://iweb.dl.sourceforge.net/project/snap7/1.3.0/snap7-full-1.3.0.tar.gz

tar -zxvf snap7-full-1.3.0.tar.gz

cd snap7-full-1.2.1/build/unix

#if you have a Raspberry Pi 2 B use this command to compile:

make –f arm_v7_linux.mk

sudo cp ../bin/arm_v7-linux/libsnap7.so /usr/lib/

sudo ldconfig

#if you have a Raspberry B,B+,A,A+ then use this command to compile:

make –f arm_v6_linux.mk all

sudo cp ../bin/arm_v6-linux/libsnap7.so /usr/lib/

sudo ldconfig

#### Download and install python-snap7

sudo pip install python-snap7

#### Now the Pi should be ready!

We are going to serve up temperature data from a DS18b20 probe connected to the pi. If you are unsure how to connected the probe see here

import time,ctypes

from ctypes import *

import logging

from threading import Thread

import snap7

import snap7.snap7types

import sys

import os

from time import sleep

logging.basicConfig()

logger = logging.getLogger()

logger.setLevel(logging.INFO)

globalData = (snap7.snap7types.wordlen_to_ctypes[snap7.snap7types.S7WLByte]*128)()

digitalOutputs = (snap7.snap7types.wordlen_to_ctypes[snap7.snap7types.S7WLByte]*16)()

digitalInputs = (snap7.snap7types.wordlen_to_ctypes[snap7.snap7types.S7WLByte]*128)()

server = None

class TempProbe(Thread):

"""

A class for getting the current temp of a DS18B20

"""

def __init__(self, fileName='',tempChangeEvent=None):

Thread.__init__(self)

self.tempDir = '/sys/bus/w1/devices/'

list = os.listdir(self.tempDir)

if(list[0][:2]=="28"):

fileName=list[0]

self.fileName = fileName

self.currentTemp = -999

self.correctionFactor = 1

self.enabled = True

self.oldTemp = 0

self.temperatureChangeEvent=tempChangeEvent

def run(self):

while self.enabled:

try:

f = open(self.tempDir + self.fileName + "/w1_slave", 'r')

lines = f.readlines()

crcLine = lines[0]

tempLine = lines[1]

result_list = tempLine.split("=")

temp = float(result_list[-1])/1000      # temp in Celsius

temp += self.correctionFactor           # correction factor

# if you want to convert to Celsius, comment this line

temp = (9.0/5.0)*temp + 32

if crcLine.find("NO") > -1:

temp = -999

self.currentTemp = temp

if self.currentTemp != self.oldTemp and self.temperatureChangeEvent:

self.oldTemp = self.currentTemp

self.temperatureChangeEvent(self.currentTemp)

except IOError as e:

print "Error: File " + self.tempDir + self.fileName + "/w1_slave" + " does not exist"

sleep(5)

pass

sleep(1)

# returns the current temp for the probe

def getCurrentTemp(self):

return self.currentTemp

def setEnabled(self, enabled):

self.enabled = enabled

def isEnabled(self):

return self.enabled

temp_probe=None

def mainloop():

server = snap7.server.Server()

global globaldata

server.register_area(snap7.snap7types.srvAreaPA, 0, digitalOutputs)     # digital outputs

server.register_area(snap7.snap7types.srvAreaMK, 0, globalData)         # internal memory

server.register_area(snap7.snap7types.srvAreaPE, 0, digitalInputs)      # digital inputs

server.start()

global temp_probe

temp_probe = TempProbe()

temp_probe.start()       # start getting temperature async

while True:

while True:

event = server.pick_event()

# write temperature to input memory Input Real 67 (IREAL67)

snap7.util.set_real(digitalInputs, 67, temp_probe.getCurrentTemp())

# fires the following when an event happens (connecting clients, read requests...)

if event:

#logger.info(server.event_text(event))

# print server.event_text(event)

print temp_probe.getCurrentTemp()

else:

break

time.sleep(.01)

server.stop()

server.destroy()

temp_probe.setEnabled(False)

temp_probe.join()

if __name__ == '__main__':

if len(sys.argv) > 1:

snap7.common.load_library(sys.argv[1])

mainloop()

temp_probe.setEnabled(False)

temp_probe.join()

#### Now run it!

#### Setting up your Wonderware and DAserver

Add a S7Cp device

Put in the Ip Address of your Raspberry pi

Create a device group. I called mine Pi

Add your device items. In this example we are doing temperature at IREAL67

Create an Access Name in your wonderware. I called mine Pi

Create a tagname and reference it to our Temperature item

attach your tagname to a text field

If you wish me to show anymore examples of how to write or read inputs or outputs in the pi using this protocol please let me know.

### Raspberry Pi SCADA Part 1, Modbus Temperature Sensor

# Raspberry Pi SCADA Part 1, Modbus Temperature Sensor.

One great thing about the Pi is that it is so cost effective in some SCADA applications. With several different languages to be able to present your data. In the many crazy off the wall things I will do in my series here I will start off with using pymodbus,the DS18b20, and a 4.7k resistor temperature probe to get building temperature. Now there are known security flaws with modbus since it is an open protocol, but with this example you can only read data from the addresses.  This will be in holding the register 0x00 and will need to be scaled by 100 afterwords.

| My total cost: |
| --- |
| Model-A Pi: | $25.00 |
| Resistors: | .05 |
| Connectors: | .20 |
| Wifi | $10.00 |
| Temp Sensor | $3.00 |
| Total | $38.25 |

### SETUP

get pymodbus and dependencies:

sudo apt-get install python-pymodbus python-twisted-conch

from pymodbus.server.async import StartTcpServer

from pymodbus.device import ModbusDeviceIdentification

from pymodbus.datastore import ModbusSequentialDataBlock

from pymodbus.datastore import ModbusSlaveContext, ModbusServerContext

from pymodbus.transaction import ModbusRtuFramer, ModbusAsciiFramer

from twisted.internet.task import LoopingCall

from threading import Thread

from time import sleep

import os

os.system('modprobe w1-gpio')

os.system('modprobe w1-therm')

temperature =0

class Temp(Thread):

"""

A class for getting the current temp of a DS18B20

"""

def __init__(self, fileName=''):

Thread.__init__(self)

self.tempDir = '/sys/bus/w1/devices/'

list = os.listdir(self.tempDir)

if(list[0][:2]=="28"):

fileName=list[0]

self.fileName = fileName

self.currentTemp = -999

self.correctionFactor = 1;

self.enabled = True

def run(self):

while True:

if self.isEnabled():

try:

f = open(self.tempDir + self.fileName + "/w1_slave", 'r')

except IOError as e:

print "Error: File " + self.tempDir + self.fileName + "/w1_slave" + " does$

return;

lines=f.readlines()

crcLine=lines[0]

tempLine=lines[1]

result_list = tempLine.split("=")

temp = float(result_list[-1])/1000 # temp in Celcius

temp = temp + self.correctionFactor # correction factor

#if you want to convert to Celcius, comment this line

temp = (9.0/5.0)*temp + 32

if crcLine.find("NO") > -1:

temp = -999

self.currentTemp = temp

#print "Current: " + str(self.currentTemp) + " " + str(self.fileName)

sleep(1)

#returns the current temp for the probe

def getCurrentTemp(self):

return self.currentTemp

#setter to enable this probe

def setEnabled(self, enabled):

self.enabled = enabled

#getter

def isEnabled(self):

return self.enabled

def updating_writer(a):

context  = a[0]

register = 3

slave_id = 0x00

address  = 0x00

#print pi.getCurrentTemp(),str(int(pi.getCurrentTemp()*10))

values = [int(pi.getCurrentTemp()*100)]

context[slave_id].setValues(register,address,values)

store = ModbusSlaveContext(

di = ModbusSequentialDataBlock(0, [0]*100),

co = ModbusSequentialDataBlock(0, [0]*100),

hr = ModbusSequentialDataBlock(0, [0]*100),

ir = ModbusSequentialDataBlock(0, [0]*100))

context = ModbusServerContext(slaves=store, single=True)

identity = ModbusDeviceIdentification()

identity.VendorName  = 'pymodbus'

identity.ProductCode = 'PM'

identity.VendorUrl   = 'http://github.com/bashwork/pymodbus/'

identity.ProductName = 'pymodbus Server'

identity.ModelName   = 'pymodbus Server'

identity.MajorMinorRevision = '1.0'

pi = Temp()

pi.start()

time = 5 # 5 seconds delaytime = 5 # 5 seconds delay

loop = LoopingCall(f=updating_writer, a=(context,))

loop.start(time, now=False) # initially delay by time

StartTcpServer(context, identity=identity, address=("localhost", 502))

#change localhost to your ip address.

Top of Form

Check out how to control Raspberry Pi outputs using this library here

Bottom of Form

### Raspberry Pi SCADA Part 2, Modbus TCP PWM Controller

Raspberr Pi SCADA Part 2, Modbus PWM Controller

Since finding a cheap alternative to PLC whilst using an industrial protocol is a popular idea the Raspberry Pi has caught many eyes on doing this.  I posted once on reading a temperature sensor and serving it up on the Pi using ModbusTCP. This time I expound on it and show you how to control something. In this case it will be a PC 12v fan.

Parts:

- 1 x Raspberry pi

- 1 x Darlington Transistor

- 1 x PC Fan (with about  500ma load)

- 1 x 12v DC Power Supply

Setup:

- Installing pymodbus and dependencies:

  - sudo apt-get install python-pymodbus python-twisted-conch

- Wire up your Fan, LED, or motor

- View my code from Github.

Video of what I did:

Source Code:

from pymodbus.server.async import StartTcpServer

from pymodbus.device import ModbusDeviceIdentification

from pymodbus.datastore import ModbusSequentialDataBlock, ModbusSlaveContext, ModbusServerContext

from pymodbus.transaction import ModbusRtuFramer, ModbusAsciiFramer

from twisted.internet.task import LoopingCall

from threading import Thread

import pid

import threading

from time import sleep

import RPi.GPIO as GPIO

import os

os.system('modprobe w1-gpio')

os.system('modprobe w1-therm')

#set up Raspberry GPIO

GPIO.setmode(GPIO.BCM)

GPIO.setwarnings(False)

GPIO.setup(25,GPIO.OUT)

pwm = GPIO.PWM(25,60)

pwmDutyCycle=100

pwm.start(pwmDutyCycle)

temperaturePoll = None

class Temp(Thread):

"""

A class for getting the current temp of a DS18B20

"""

def __init__(self, fileName=''):

Thread.__init__(self)

super(Temp, self).__init__()

self._stop = threading.Event()

self.tempDir = '/sys/bus/w1/devices/'

list = os.listdir(self.tempDir)

if(list[0][:2]=="28"):

fileName=list[0]

self.fileName = fileName

self.currentTemp = -999

self.correctionFactor = 1;

self.enabled = True

self.Run=True

def run(self):

while self.isEnabled():

try:

f = open(self.tempDir + self.fileName + "/w1_slave", 'r')

except IOError as e:

print "Error: File " + self.tempDir + self.fileName + "/w1_slave" + " does not exits"

return;

lines=f.readlines()

crcLine=lines[0]

tempLine=lines[1]

result_list = tempLine.split("=")

temp = float(result_list[-1])/1000 # temp in Celcius

temp = temp + self.correctionFactor # correction factor

#if you want to convert to Celcius, comment this line

temp = (9.0/5.0)*temp + 32

if crcLine.find("NO") > -1:

temp = -999

self.currentTemp = temp

#print "Current: " + str(self.currentTemp) + " " + str(self.fileName)

sleep(.5)

#returns the current temp for the probe

def getCurrentTemp(self):

return self.currentTemp

#setter to enable this probe

def setEnabled(self, enabled):

self.enabled = enabled

#getter

def isEnabled(self):

return self.enabled

def updating_writer(a):

context  = a[0]

register = 3

slave_id = 0x00

address  = 0x00

global pwmDutyCycle,temp

#uncomment to debug temperature

print temp.getCurrentTemp()

values = [int(pwmDutyCycle),temp.getCurrentTemp()*100]

context[slave_id].setValues(register,address,values)

def read_context(a):

context  = a[0]

register = 3

slave_id = 0x00

address  = 0x00

value = context[slave_id].getValues(register,address)[0]

global pwmDutyCycle

if(value!=pwmDutyCycle):

print value

pwmDutyCycle=value

pwm.ChangeDutyCycle(pwmDutyCycle)

def main():

store = ModbusSlaveContext(

di = ModbusSequentialDataBlock(0, [0]*100),

co = ModbusSequentialDataBlock(0, [0]*100),

hr = ModbusSequentialDataBlock(0, [0]*100),

ir = ModbusSequentialDataBlock(0, [0]*100))

context = ModbusServerContext(slaves=store, single=True)

identity = ModbusDeviceIdentification()

identity.VendorName  = 'pymodbus'

identity.ProductCode = 'PM'

identity.VendorUrl   = 'http://github.com/simplyautomationized'

identity.ProductName = 'pymodbus Server'

identity.ModelName   = 'pymodbus Server'

identity.MajorMinorRevision = '1.0'

time = 5 # 5 seconds delaytime = 5 # 5 seconds delay

writer = LoopingCall(read_context,a=(context,))

loop = LoopingCall(updating_writer, a=(context,))

loop.start(.5) # initially delay by time

writer.start(.1)

StartTcpServer(context, identity=identity)#, address=("localhost", 502))

#cleanup async tasks

temp.setEnabled(False)

loop.stop()

writer.stop()

GPIO.cleanup()

if __name__ == "__main__":

temp = Temp()

temp.start()

main()

### Raspberry Pi SCADA: Communitating with S7-200 Using Python

Video Demo

For how to compiling snap7 on the pi click here

Since the S7-200 is a bit different then the newer S7-1500/S7-1200 PLCs it took a bit of work to get it working. The python-snap7 library was missing the connection method for the old TSAP method of connecting. So I forked the library and added the missing function to connect. I did a pull request to get it merged back with the original and it's pending currently. Here is the link to the forked library:
https://github.com/SimplyAutomationized/python-snap7

I've also been working on a din rail enclosure for the pi that wasn't so large.   I'm going to be adding a 24vdc to 5vdc dc-dc converter to the enclosure it so I don't have to bring in usb to it.

Anyway back to the good stuff. I added the Cli_Connection in the Client.py library so the code will now be as follows:

import snap7 as p

plc = p.client.Client()

plc.set_connection_params('10.10.55.250',11,11)

plc.connect()

print plc.get_connected()

if(plc.get_connected()):

plc.disconnect()

If you import my helper S7200.py library from github the code is as follows:

import S7200 as p

plc = p.S7_200('10.10.55.250',0x1100,0x1100,debug=True)

plc.writeMem('QX0.0',True)

print plc.getMem('freal10')

plc.writeMem('freal10',3.141592653589)

print plc.getMem('freal10')

print plc.getMem('QX0.0')

- This will turn on output 1 (Q0.0, will turn on if there's no ladder associated with it).

- Then read the real number stored in V memory 10.

- Then write pi to it ;-).

- Then read from it again.

- Then read if output 1 (Q0.0) is on still\

Github for helper library found here:
https://github.com/SimplyAutomationized/raspberrypi/blob/master/S7-200pi/S7200.py